RECEIVER HITCH

Application

2007-15	IQ/TOURING HITCH/TURBO/600 LXT

IMPORTANT: We highly recommends that the installation be performed by a certified technician.

Before you begin, read these instructions and check to be sure all parts and tools are included

Please keep these instructions for future reference.

| This kit includes: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Qty |  | Part Description |  | OEM reference |
| 1 |  | Weld Hitch Black |  | 1014685-067 |
| 4 |  | Rivet |  | 7621418 |
| 10 |  | Rivet |  | 7661637 |
| 1 |  | Instructions |  | 9920810 |

You will need to Supply:

Drill and #7 or 13/64" drill bit			1/8” Punch		Ratchet and 7/16” socket

Rivet gun							Torx T-20

IMPORTANT : For your safety, and to ensure a satisfactory installation, perform all installation steps correctly in the sequence as shown on the attached instructions.

INSTALLATION INSTRUCTIONS

1. Remove taillight housing by removing six Torx

screws from inside rear wall of trunk area.

Remove two push-on fastener clips from under

the taillight assembly, one from each side.

Carefully disconnect wire harness from taillight

in two places.

2. Using a 1/8" punch, tap out all the snow flap

rivet nails. Drill and remove four rivets holding

snow flap onto black bracket and rear close off

panel. (Photo 2)

3. Using a 1/8” punch, tap out the six accessible

cooler rivet nails holding cooler onto tunnel

extension. (Photo 2)

4. Remove the four bolts, two on each side,

holding bracket to bumper using ratchet and

socket. (Photo 3)

5. Cooler should now be attached only by hoses

at each end. Swing cooler down out of the way

to access the remaining two rivets holding the

bracket to tunnel extension. Drill and remove

these two rivets.

6. Drill and remove four rivets holding brackets to

top of tunnel extension. Brackets should now be

able to be removed from machine. Do not re-use

these brackets. (Photo 4)

7. Position hitch in location where brackets were

removed, making sure receiver is on bottom side

of bar connecting brackets. Install four bolts

through bumper side and into hitch side plates.

Torque to 10 ft. lbs. (Photo 5)

8. Install rivets and bolts in opposite order they were

removed. Align holes at each step before installing

rivets to ease installation. Reinstall bumper using

original hardware and torque to 10 ft. lb.

9. Attach wire harness connections to taillight

assembly. Reinstall taillight assembly and tighten

screws to 12 in. lb.

DISCLAIMER: This item is sold without any expressed or implied warranty of merchantability or fitness for a particular purpose. The Manufacturer, Distributor, nor Dealer shall be held liable for any special, incidental, or consequential damages, including damage, loss of revenue, cost purchased or use of this product.