DIRECTIONS:

- Find and remove the item that caused the puncture in the tire.

- Apply rubber solution on the Reamer Tool; push Reamer Tool into puncture and work up and down until the Reamer Tool moves easily in the hole. This cleans and clears the puncture.

- Thread the tire seal/rubber plug into the Hook Tool until one-half of the tire seal/rubber plug is sticking from both ends.

- Push the tire seal/rubber plug into the hole until 1/2” of the tire seal/rubber plug remains outside the tire, then turn the Hook Tool 1/4 turn and pull out slowly.

- Cut excel tire seal/rubber plug as close to the tire as possible.

- If puncture is too big for the tire seal/rubber plug to repair, please seek a professional repair shop.

- Hook Tool

CO2 CANISTER WARNING

- Don’t inhale.

- Don’t remove canister before empty.

- Don’t leave canister sitting in the sun (don’t exceed 113°F/45°C).

- Store in cool/dry place.

- When the canister starts to discharge, the canister will instantly drop to -64°F/-53°C, so put CO2 canister into the provided covering or wrap canister with a rag before using to avoid frostbite.

- Once empty, please recycle empty cartridges.

CO2 INSTRUCTOINS:

- This canister contains natural/pure CO2.

- Each canister contains 16g of liquid CO2.

How to use:

- Put the CO2 canister into the covering or wrap canister with a rag.

- Check the style of tire valve, then decide which adaptor to use.

- Remove the tire valve cap, attach and tighten the adaptor to the valve as shown in the illustration. Link the other side with the CO2 canister slowly.

- After filling the tire the desired amount, remove the valve adaptor side from the tire first (don’t remove the canister side before removing the valve side, otherwise gas will escape from the tire).

- Replace the tire valve cap and check the air pressure.

MANUFACTURED IN TAIWAN