SEAT FOAM REPAIR KIT

- Vinyl specially formulated for use on seats; superior to what you'd find in upholstery shops

- UV and weather resistant

- Color and grain compliments most seats

SEAT COVER VINYL

- Contains premium high-density seat foam, a sheet of low-density cover foam

- Weather resistant, durable foam guards against premature wear

General Seat Repair Instructions

Step 1: Remove seat

Seats are usually fastened with a few bolts in the rear and held down with a tab up front near the fuel tank. Remove the hardware and pull the seat .

**Some snowmobile applications the tank and seat base are one piece. In this case, the fuel tank needs to be drained and fittings plugged so residual fuel does not leak out during the re-covering process.

Step 2: Take off seat cover

With the seat assembly off of the machine, pry the staples with a small, flat screwdriver and pull them with a pliers.

On some applications The cover may also be attached with straps that hold it against the hump at the back, also called the seat bun. Unhook the straps as necessary and be sure to remove all of the old staples so they don’t damage the new cover.

Step 3: Repair foam

With the bare seat foam exposed, cut out the needed portion of foam approximately 1” deep . Be sure to take enough around effected area to avoid further tears. *For better results try to cut a square shape.

Using the foam repair kit high-density foam cut a similar size chunk and apply some permanent fabric adhesive or another permanent, flexible, waterproof adhesive.   Then position replacement foam in the cavity on seat foam and connect the foam making sure to hold together long enough to bond together.

If using low-density foam follow same instructions.

Step 4: Set new Cover - 2 options. Patch the area or replace the entire cover.

- Using a replacement cover - Position the new cover on the cushion and get it squared up. Align the seams with the corners and edges of the seat foam and continue to step 5.

  - If necessary pull the cover straps through the seat bun with a long needlenose or duckbill pliers and position the cover so it lines up with the trunk access hole

- Using seat cover vinyl  –  you will need to stitch together the cover according to the cover you removed.

- Using a patch – remove seat cover per above instructions. Cut a portion of cover material large enough        to extend at least 2”  beyond the tear in all directions. Stitch the patch on cover and replace. We recommend using tough all weather thread and double stitching. Then replace seat cover.

Step 5: Replace Seat Cover -

Working with the seat on a clean work area free of staples, center the cover at the front of the seat and using a pneumatic stapler put a few staples there to hold its position. Then do the same in the rear to get the rear centered or trunk hole centered. Put these first staples as close to the edge of the material as possible; this way staple holes won’t show in case you need to remove them and re-position the cover. Now that the cover is tacked in place with a few staples, make sure seams are lined up correctly and that the corner seams in the fabric are at the corner of the base. Continue stapling keeping about ½” between staples. Its important to keep constant tension on the material while doing this.